Distribution Layer Switches

Background:

As part of the NAIT ICTC Network Design Requirements, Mantacore was contracted to spec out distribution-layer devices for the second floor of the ICTC Center. These include switches and wireless access points. The requirements for distribution-layer devices are as follows:

Availability:


The key to high availability is redundancy. All distribution-layer devices will have a redundant backup. All distribution-layer devices will be linked to core-layer devices via redundant links. The OSPF routing protocol will be used to enable quick convergence times in the event of a failure.

Performance:


All distribution-layer devices will be interconnected with each other, access-layer devices, and core-layer devices via high-speed 1000BaseT links.

Manageability:


All distribution-layer devices must be SNMP-capable, to enable remote management.

Security:


Switches will be selected over hubs to prevent traffic from being intercepted by third parties, as well as for performance reasons. VLANs will be used to isolate classrooms. Access Control Lists will be used to restrict access to workstations and to prevent IP spoofing.

Affordability:


Devices must be as inexpensive as possible.

From the above goals, the following requirements have been determined:

Distribution-Layer Switches:

1. Must support VLANs

2. Must have minimum 11 1000BaseT ports (1 to connect each switch to it's redundant backup, 2 to connect to the core layer, 8 to connect to access layers)

3. Must support IEEE 802.1q trunking

4. Must support SNMP management protocol

5. Must support IP routing

6. Must support OSPF routing protocol

7. Must support Access Control Lists

8. Want affordability

Decision Matrix:


[image: image1.wmf]Catalyst 3750G-12S 

Yes

Yes

Yes

Yes

2. Ports

24x0/100/1000

24x0/100/1000

24x0/100

12xGigabit SFP

3. 802.1q

Yes

Yes

Yes

Yes

4. SNMP

Yes

Yes

Yes

Yes

5. IP Routing

Yes

Yes

Yes

Yes

6. OSPF

Yes

No

Yes

Yes

Yes

Yes

Yes

Yes

8. Cost

$14,486.00

$8,693.00

$8,686.00

$17,386.00

Meet/Exceed/Fail

Exceed

Fail

Fail

Fail

Cisco Catalyst 3750G-24T-E

Cisco Catalyst 3750G-24T-S 

Cisco Catalyst 3750-24TS-E 

1. VLANs

7. ACLs


Final Decision:

Cisco Catalyst 3750G-24T-E

_174837820.unknown

